King Abdulaziz University

Faculty of Meteorology, Environment and Arid Land Agriculture

Department of Hydrology and Water Resources Management

Mar. 22, 2009                                                                                                                     2ed Term 29/30

HWR 443

Design of Urban Water Networks

Assignment No. 2

- Using Hazen-Williams equation, determine the minimum diameter of a new cast iron pipe for the following conditions: the upstream end is 56.3 m higher than the downstream end, which is 3.75 km away.  The upstream pressure is 500 kPa and the desired downstream pressure and flow rate are 390 kPa and 9750 l/min., respectively.  What is the minimum diameter needed if pipes are available in 50 mm increments.

- A flow of 2700 gpm is driven through a level 370 yard long, 10 in diameter cast iron pipe to a large industry site.  If the pressure at the upstream end of the pipe is 75 psi, what will be the pressure at the industry?

- If the industry complains that, the flow rate is changing between 1900 and 2900 gpm during the day.  What is the expected upstream pressure range?  Draw a rating curve between the flow rate and the upstream pressure.

- An engineer suggested replacing the 10 in diameter pipe with a 8 in PVC pipe, what would be the required upstream pressure if the downstream pressure is kept unchanged.