King Abdulaziz University

Faculty of Meteorology, Environment and Arid Land Agriculture

Department of Hydrology and Water Resources Management

1st Term 1430/1431 							               1/1/1431

### HWR 422

Hydraulic Structures

Exercise 5

1. Design a rectangular sharp crested weir according to the following data;

Q = 0.65 m3/sec ;  B = 1.2  m;  d = 1.0 m

2. Determine the flow over a rectangular sharp crested weir if the head over the crest is 0.52 m. The weir has the following characteristics;

bc= 0.65 m;  P1 =O.32  m;  B = l.0  m

3. Design a V -notched sharp crested weir according to the following data:

Q = 0.35  m3/sec:  B = 1.3 m;   d = 0.9 m

4. Determine the flow over a V-notched sharp crested weir if the head over the crest is 0.52 m. The weir has the following characteristics;

P1 = 0.35  m;  B  =  1.1 m

5. Design a broad crested weir according to the following data:

Qmax = 6.0 m3/sec; B = 4.0 m;   d = 1.8 m

# Hence evaluate the discharge for a head of 0.45 m.