King Abdulaziz University

Faculty of Meteorology, Environment and Arid Land Agriculture

Department of Hydrology and Water Resources Management

1stTerm 1430/1431							         	15/1/1431

### HWR 422

Hydraulic Structures

Exercise 6

1. Design an Ogee type weir to allow a maximum discharge of 70 m3/sec. The crest width is 25.0 m and the height of the weir is 10.0 m.

2. Determine the opening of a gate that controls the discharge in a channel of width 10.0 m and a water depth of 2.7 m if the maximum discharge is 15 m3/sec.

3.  Design a stilling basin according to the following data:

Q = 65 m3/sec;  B= 25 m;  Z = 16 m